Team Members:

- M Praveen kumar CB.EN.U4CSE20449

- S Jayandhar CB.EN.U4CSE20425

- M Abishek CB.EN.U4cse20402

Assembly Code:

AREA bubble,CODE,READONLY

ENTRY

Main

LDR R0,=0X40000000

MOV R1, #6 		;Index	value

STR R1, [R0],#4	;Stores in memory

MOV R1, #5

STR R1, [R0],#4 ; memory auto increments by 1

MOV R1, #4

STR R1, [R0],#4

MOV R1, #3

STR R1, [R0],#4

MOV R1, #1

STR R1, [R0],#4

MOV R1, #0

STR R1, [R0],#4

MOV R2,#6		;R2 has the size of the array

MOV R3,#0		;R3 is the outer loop index

MOV R4,#0		;R4 is the inner loop index j

Outer

LDR R0,=0X40000000

CMP R3, R2

BGT Exit

MOV R4,#0

SUB R5,R2,R3

SUB	R5,R5,#1	;Inner loop break condition

BL Inner

ADD R3,R3,#1

B Outer

Inner

CMP R4, R5

BXGE LR

LDR R6,[R0],#4

LDR R7,[R0]

CMP R6, R7

MOVGT R8,R6		;Swap occurs when arr[c]>arr[c+1]

MOVGT R6,R7

MOVGT R7,R8

STR R7,[R0]		;Storing  updated register

STR R6,[R0,#-4]

ADD R4,R4,#1

B Inner

Exit B Exit

END

Insertion Code:

AREA    DATAARR, DATA, READWRITE

ARRAY	DCD 8,2,3,0,1,4,9,5,6,7		;Array for sorting

AREA    MYCODE, CODE, READONLY

ENTRY

EXPORT main

main

MOV	R0, #1			;Start index

LSL R0, R0, #2

MOV	R1, #10			;Size of array

LSL R1, R1, #2

BL	INSERTION

LDR R0, =ARRAY

LDR R1, [R0, #0]

LDR R2, [R0, #4]

LDR R3, [R0, #8]

LDR R4, [R0, #12]

LDR R5, [R0, #16]

LDR R6, [R0, #20]

LDR R7, [R0, #24]

LDR R8, [R0, #28]

LDR R9, [R0, #32]

LDR R10, [R0, #36]

B 	STOP

INSERTION

SUB	SP, SP, #12		;Mov stack for keep current parameter

STR	LR, [SP, #8]	;Save return address

STR	R0,	[SP, #4]	;Save start index argrument

STR	R1, [SP, #0]	;Save size of array argrument

CMP	R0, R1			;If start index = size of array

BEQ	RET				;Return to main

CMP	R0, #0			;If start index = 0

BEQ	RET				;Return to main

;Check if less than after

LDR	R12, =ARRAY		;Get address of array

LDR	R2, [R12, R0]	;Load array[start] to R2

SUB R4, R0, #4		;Get value of start-1 in R4

LDR R3, [R12, R4]	;Load array[start-1] to R3

CMP	R2, R3	 		;if array[start]<array[start-1]

BLT	SWAP			;swap it

RECUR

LDR R0, [SP, #4] 	;Load start index from this recursion

LDR R1, [SP, #0]	;Load size from this recursion

ADD R0, R0, #4		;increse start

BL INSERTION		;call recursion for next number

B 	RET 			;return when end function

SWAP

MOV R5,	R2			;temp = array[start]

STR R3,	[R12, R0] 	;array[start] = array[start-1]

STR R5, [R12, R4]	;arrar[start-1] = temp

MOV R1, R0			;pass start to size

SUB R0, R0, #4		;decrese start

BL INSERTION 		;call recursion for prev number

B	RECUR 			;back to current function call

RET

LDR LR, [SP, #8] 	;load LR to return caller

ADD SP, SP, #12 	;pop stack

MOV PC, LR 			;set LR to PC

STOP

B STOP

END